CO5 – AT2: Pseudocode Writing Task — Answers

# 1. Real-Time Face Detection and Recognition

## Pseudocode

BEGIN

INITIALIZE video_capture <- OpenCV.VideoCapture(camera_source)

LOAD trained_face_database <- LoadRegisteredFaceEmbeddings("database_path")

LOAD face_detector_model <- LoadDNNFaceDetector()

LOAD embedding_model <- LoadFaceEmbeddingModel()

SET frame_skip <- N          // process every Nth frame (real-time)

SET frame_count <- 0

SET similarity_threshold <- T

SET previous_detections <- EMPTY LIST

WHILE video_capture.isOpened() DO

frame <- video_capture.readFrame()

IF frame is NULL THEN

BREAK                          // end of stream / camera error

END IF

frame_count <- frame_count + 1

// ---- Preprocessing ----

gray_frame <- ConvertToGrayscale(frame)

normalized_frame <- ApplyCLAHE(gray_frame)     // illumination correction

denoised_frame <- GaussianBlur(normalized_frame)

// ---- Efficient loop control: skip heavy work on some frames ----

IF (frame_count MOD frame_skip != 0) THEN

DisplayFrame(frame, previous_detections)  // reuse last boxes

IF UserPressedQuitKey() THEN BREAK

CONTINUE

END IF

// ---- Face Detection (handles multiple faces) ----

face_boxes <- face_detector_model.Detect(denoised_frame)

detections <- EMPTY LIST

FOR EACH box IN face_boxes DO

face_roi <- CropRegion(frame, box)

aligned_face <- AlignFace(face_roi)         // eye-line normalization

// ---- Feature Extraction ----

embedding <- embedding_model.Extract(aligned_face)

// ---- Recognition ----

best_match <- NULL

best_distance <- INFINITY

FOR EACH (identity, stored_embedding) IN trained_face_database DO

distance <- ComputeDistance(embedding, stored_embedding)

IF distance < best_distance THEN

best_distance <- distance

best_match <- identity

END IF

END FOR

IF best_distance < similarity_threshold THEN

label <- best_match

ELSE

label <- "Unknown"

END IF

APPEND (box, label) TO detections

END FOR

previous_detections <- detections    // cache for reuse on skipped frames

// ---- Visualization ----

FOR EACH (box, label) IN detections DO

DrawRectangle(frame, box)

DrawLabel(frame, label, box.top_left)

END FOR

DisplayFrame(frame)

// ---- Loop termination ----

IF UserPressedQuitKey() THEN

BREAK

END IF

END WHILE

video_capture.Release()

CloseAllDisplayWindows()

END

## How the Constraints Are Met

- Real-time operation: the main loop reads and processes frames continuously with no blocking waits, and heavy detection/recognition work is skipped on non-key frames (frame_skip), reusing the previous frame's results for display.

- Minimized processing delay: only every N-th frame runs full detection + recognition; grayscale conversion and CLAHE keep preprocessing lightweight; the loop exits immediately via a quit-key check or on stream end.

- Multiple faces handled simultaneously: the detector returns a list of all face boxes per frame, and the FOR EACH loop extracts features, recognizes, and labels every detected face independently within the same frame pass.

# 2. Vehicle Detection, Tracking and Counting

## Pseudocode

BEGIN

INITIALIZE video_capture <- OpenCV.VideoCapture(traffic_camera_source)

LOAD vehicle_detector_model <- LoadDNNVehicleDetector()

INITIALIZE tracker_list <- EMPTY LIST         // active vehicle trackers

INITIALIZE next_track_id <- 0

INITIALIZE vehicle_count <- 0

INITIALIZE counted_ids <- EMPTY SET           // prevents double counting

DEFINE counting_line <- ((x1, y1), (x2, y2))

DEFINE roi_region <- predefined polygon        // region of interest

DEFINE MAX_MISSED_FRAMES <- K

WHILE video_capture.isOpened() DO

frame <- video_capture.readFrame()

IF frame is NULL THEN

BREAK                    // end of stream

END IF

// ---- Preprocessing ----

gray_frame <- ConvertToGrayscale(frame)

blurred_frame <- GaussianBlur(gray_frame)

roi_frame <- MaskRegionOfInterest(frame, roi_region)

// ---- Detection ----

detected_boxes <- vehicle_detector_model.Detect(roi_frame)

// ---- Predict existing tracks forward ----

FOR EACH tracker IN tracker_list DO

tracker.PredictNextPosition()   // e.g., Kalman filter predict step

END FOR

// ---- Associate detections to existing tracks (IOU-based matching) ----

matched_pairs, unmatched_detections, unmatched_trackers <-

AssociateDetectionsToTrackers(detected_boxes, tracker_list)

// ---- Update matched trackers ----

FOR EACH (detection, tracker) IN matched_pairs DO

tracker.Update(detection)

tracker.missed_frames <- 0

END FOR

// ---- Create new trackers for unmatched detections ----

FOR EACH detection IN unmatched_detections DO

new_tracker <- CreateTracker(detection, next_track_id)

next_track_id <- next_track_id + 1

APPEND new_tracker TO tracker_list

END FOR

// ---- Age out trackers with no matching detection ----

FOR EACH tracker IN unmatched_trackers DO

tracker.missed_frames <- tracker.missed_frames + 1

IF tracker.missed_frames > MAX_MISSED_FRAMES THEN

REMOVE tracker FROM tracker_list

END IF

END FOR

// ---- Counting logic: single-crossing check per vehicle ----

FOR EACH tracker IN tracker_list DO

current_centroid <- tracker.GetCentroid()

previous_centroid <- tracker.GetPreviousCentroid()

IF HasCrossedLine(previous_centroid, current_centroid, counting_line) THEN

IF tracker.id NOT IN counted_ids THEN

vehicle_count <- vehicle_count + 1

ADD tracker.id TO counted_ids     // avoids double counting

END IF

END IF

END FOR

// ---- Visualization ----

FOR EACH tracker IN tracker_list DO

DrawRectangle(frame, tracker.GetBoundingBox())

DrawLabel(frame, "ID:" + tracker.id, tracker.GetBoundingBox().top_left)

END FOR

DrawLine(frame, counting_line)

DisplayText(frame, "Vehicle Count: " + vehicle_count)

DisplayFrame(frame)

// ---- Efficient loop termination ----

IF UserPressedQuitKey() OR EndOfStream(video_capture) THEN

BREAK

END IF

END WHILE

video_capture.Release()

CloseAllDisplayWindows()

PRINT "Final Vehicle Count: " + vehicle_count

END

## How the Constraints Are Met

- Avoiding double counting: each tracker has a persistent unique ID; a counted_ids set records which IDs have already been counted, so a crossing event for the same tracker ID is never counted twice, even if the vehicle lingers near the line across several frames.

- Handling multiple vehicles simultaneously: tracker_list holds one tracker per vehicle; detections are matched to trackers each frame via ID-based (IOU) association, so every vehicle is independently tracked, counted, and displayed in the same pass.

- Near real-time performance: detection is restricted to the region of interest (roi_frame) to reduce processed area; Kalman-style prediction lets trackers persist through brief missed detections without expensive re-detection; and stale trackers are removed after MAX_MISSED_FRAMES to keep the active tracker list small and the loop efficient. The loop terminates cleanly on a quit key or end of stream.